Key to 21st April AC40DBFD specimen

- Access panel frame manufactured from 1.2mm thick Zintec steel sheet. The 25mm picture frame surround was mitred at each corner. All seems were fully welded.

- Access panel door tray manufactured from 0.9mm thick Zintec steel sheet, cut and folded.

- 12.5mm thick Lafarge Megadeco wallboard factory screw fixed to the rear of the door tray using 25mm drywall screws.

- 3 point locking system with central lock operating 8mm diameter rods which lock into frame sides. The rods are secured with 4no guide blocks.

- 2 point locking system with central lock operating 8mm diameter rods which lock into frame sides. The rods are secured with 4no guide blocks.

- 50mm thick E-Coustiquilt matting inserted into cavities between door tray and Megadeco wall board.

- Mild steel continuous hinge welded to door tray and bolted to frame using M6 bolts, nuts and washers.

- 4no 10mm wide x 2mm thick self adhesive intumescent (BS476, part 1) glazing strip (Envirograph Ref: G10/10) fitted around inside edge of frame and rear of rear protection plate.

- 2no self adhesive Lorient Neoprene Seal fitted around inside edge of frame.

- 1.5mm Zintec thick door angle 65mm wide.

- 14mm diameter metal screw in bung with Neoprene washer covering lock hole in door tray.

- Handle with 8mm square bar fixed to the 2 point locking system, screw fixed to the rear protection plate.

- 1.5mm thick Zintec steel rear protection plate screw fixed to rear of door tray.